CAN DO HOME SERVICES

PROFESSIONAL HOME INSPECTION

March 6, 2012

Visual Property Inspection

701 5th Ave

New Hyde Park, NY 11040

Prepared for:

Bill Hees

13 Cross St., Belmont, Ma 02478

Inspected by :

BOB VAN STRY

Lic.# 1600005960

34 Metzner Rd.,

Ronkonkoma, New York 11779

Phone : (631) 467-8900

e-mail : candohomeinsp@aol.com

PROPERTY AND SITE

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

____________________________________________________________________________________________________

LIMITATION:          Restricted / No Access               Debris / Obstruction               Snow / Ice Cover               Vegetation/Tree/Shrub

CONDITIONS:               Clear / Sunny               Cloudy / Overcast               Rain / Snow               Approx. Temperature

Wet / Swampy / Damp / Dry / Frozen Ground                                           Approx. Hydrant Distance

_________________________________________________________________________________________________________________

BUILDING               Approx. 90 Years Old

Traditional          Cape Cod          Colonial          Hi-Ranch          Dutch Colonial          Ranch          Bungalow          Victorian

Rural         Bi-Level          Tri-Level          Addition          Condo          Row / Town          Other          2 Story          3 Story

Items like in-ground sprinkler system, above ground pool and components, hot tub and deck wet bar were not checked nor inspected as per our agreement.

Visible evidence indicates an acceptable level of construction.

No movement is evident.

Inspection of the structural components was restricted due interior finishes.

_________________________________________________________________________________________________________________

LANDSCAPING               Slopes to House                                                                                                                           FUNCTIONAL

Ravine          Site Erosion          Flower Bed          Hedge / Shrub          Tree / Vine          Earth to Wood          No Swale          Other

_________________________________________________________________________________________________________________

DRIVEWAY                         Slopes to House                                                                                                                 DAMAGED—YES

Paving Stone          Gravel          Concrete          Asphalt          Dirt          Parking Area          None

Repair to the concrete will be needed.

_________________________________________________________________________________________________________________

WALKWAY/PATH            Slopes to House                                                                                                                    DAMAGED—NO

Paving Stone          Gravel          Concrete          Asphalt          Brick          Slate          Dirt          Other

_________________________________________________________________________________________________________________

PORCH                                   Unsecured                                                                                                                           DAMAGED—NO

Metal          Wood          Concrete          Brick          Block          Slate

Spalling          Crack          Corrosion          Rot          Settlement          Repaint / Stain          Weather Seal          Caulk

Enclosed front porch.

Front steps looked ok. Some repointing will be needed.

_________________________________________________________________________________________________________________

LIGHTING                                                                                                                                                              OPERATIONAL—YES

None          Unsecured          Not Tested

___________________________________________________________________________________________________

RECEPTACLE                                                                                                              OPERATIONAL—YES

No Cover          Damaged          GFCI

_________________________________________________________________________________________________________________

RAILING                                Unsecured                                                                                                                                 FUNCTIONAL

Metal          Wood          PVC          Incomplete          None

Metal handrail is getting loose.

_________________________________________________________________________________________________________________

DECK / PATIO             Unsecured           Slopes to House                                                                                                         N/A

Composite          Wood          Brick          Block          Concrete          Metal          Trex          Slate

Paving Stone          Patio Block          Stone

Crack          Settlement          Spalling          Deterioration          Mold          Mildew          Rot

__________________________________________________________________________________________________

RETAINING WALL                Not Stable                                                                                                                                             N/A

No Weep Holes          Wood          Concrete          Stone          Brick

Crack          Spalling          Deterioration          Rot          Leans

EXTERIOR

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

______________________________________________________________________________________________________________

LIMITATION:          Clearance          Seasonal Storm Windows          Debris / Obstruction          Shrub / Tree / Hedge / Vines / Ivy

Snow / Ice Cover          Restricted / No Access          Parged          Other

______________________________________________________________________________________________________________

FOUNDATION WALL          Not Exposed          Limited Exposure                                                                      FUNCTIONAL

Poured Concrete          Block          Brick          Stone

Exterior Rigid Insulation          PWF / Wood          Piling / Pier          Cracking / Spalling          Mildew / Mold

Stain / Rot          Frost Heave

Parging on foundation wall conceals condition of walls. Some cracks were seen. Front corner will need repair where wood is coming off the concrete block corner.

Regrading away from foundation is needed in some areas to help keep water away. Fill and seal foundation cracks to reduce potential water penetration and subsequent damages.

_______________________________________________________________________________________________________________

WALL SURFACE                  No Ground Clearance                                                                                                 DAMAGED—NO

Aluminum          Composite / Concrete          Brick / Block          Stone          Stucco          Vinyl Siding          Wood Siding

Steel / Shingle           Split / Loose          Repoint          Repaint          Recaulk

Crack / Spalling          Mildew / Mold          Stain / Rot          Blister / Peel

Wood Siding is old . Some areas are worn more than others. Siding repair or replacement will be needed soon.

_______________________________________________________________________________________________________________

Windows                                 Inspected with Binoculars                                                                                               DAMAGED—NO

Storm          Unsecured          Repaint          Recaulk          Weather-strip          Mildew / Mold          Stain / Rot          Poor Trim

Old wood windows. Some panes were cracked. Some rot  to trim was seen. Basement windows were in poor condition and need replacing.

________________________________________________________________________________________________________________

WINDOW WELL          Improper Drainage          Well Covers          Clean 						N/A

Should have window wells along the side of the driveway. To help keep water away from the foundation.

________________________________________________________________________________________________________________

DOORS           Binds          Damaged                                                                                                                        DAMAGED—NO

Storm          Unsecured          Repaint          Recaulk          Weather-strip          Mildew / Mold          Stain / Rot          Split / Dent

Exterior doors  were old and getting  in poor condition. They need better weatherproofing and adjustment.

__________________________________________________________________________________________

LIGHTING                                                                                                                     OPERATIONAL—YES

None          Unsecured          Not Tested

____________________________________________________________________________________________________

ROOF STRUCTURE

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

_______________________________________________________________________________________________________________

INSPECTED BY:          Binocular          Roof Edge          Walk On          No Access

LIMITATION                     Deck / Patio          Gravel / Flat          Height          Steep Slope          Rain / Snow / Ice Cover

Solar Panel          Fragile          Other

Because of height and steep slope I could not walk on this roof.

________________________________________________________________________________________________________________

MAIN ROOF          Flat          Gable          Valley          Hip          Shed          Other          Poor Pitch                    Est. Age

________________________________________________________________________________________________________________

GUTTER / DOWNSPOUT                    Unsecured                                                                                                         FUNCTIONAL

TYPE          Aluminum          Galvanized          Copper          Plastic

Incomplete          Dent / Split          Corrosion          Leak          Gutter Covers

DRAINAGE          Above Ground          Below Ground          Spill / Roof          Extend Leader          Redirect Leader          Clean

Clean and maintain gutter system regularly to reduce chance of water damages as a result of clogging. Extend all downspouts minimum 6' away from foundation to reduce wall deterioration, potential water entry and subsequent damages.

________________________________________________________________________________________________________________

FASCIA / SOFFIT                     Not Vented                                                                                                                  DAMAGED—NO

Aluminum          Wood          Vinyl          Other          Loose          Mildew / Mold          Stain / Rot          Corrosion

_______________________________________________________________________________________________________________

COVERING          NUMBER OF LAYERS   1                                                                                                                   DAMAGED—NO

Asphalt Shingle          Fiberglass Shingle          Concrete / Clay Tile          Wood Shingle          Wood Shake          Tar / Gravel          Metal

Nail Exposed / Pop          Loose / Missing          Broken          Crack          Patched          Mildew / Mold          Fungus / Moss          Worn

Curl / Claw          Stain / Rot          Improper Installation

LIFE EXPECTANCY                    Typical          Middle          End          Exceeded

Shingles are functioning as intended.

Monitor seasonally for loose shingles, wind damage and deterioration.

________________________________________________________________________________________________________________

ACCESSORY               Unsecured                                                                                                                             DAMAGED—NO

Air Vent          Exhaust Vent          Vent Stack          Turbine          Electric Mast          Solar Panel          Skylight          Dish          Antenna

________________________________________________________________________________________________________________

FLASHING                    Not Checked / Concealed                                                                                                           DAMAGED—NO

Chimney          Dormer          Drip Edge          Flat Roof          Skylight          Roof to Wall          Stack          Valley          Roll Roofing

Aluminum          Galvanized          Copper / Lead          Rubber

Gap          Loose          Crack          Deterioration          Corrosion          Tarred          Patched

Reseal          Recaulk          Improper          Replace When Re-Roofing

________________________________________________________________________________________________________________

CHIMNEY / VENT                    Leaning                                                                                                                        DAMAGED—NO

Fireplace          Furnace          Gas Insert          Other

Brick          Block          Stone          Metal          Wood          Stucco          Chase

Crack          Spalling          Deterioration          Corrosion          Loose          Abandoned          No Wind Cap          Metal Liner Required

Repointing and repair to the cracks is needed at base of chimney by the boiler. Have a mason give estimate to repair.

________________________________________________________________________________________________________________

CHIMNEY CAP                    None                                                                                                                                        NOT VISIBLE

Concrete          Masonry          Metal          Other

Crack          Spalling          Deterioration          Corrosion          Loose

Edge of cap looks worn. May need some repair. Have a chimney company check further.

________________________________________________________________________________________________________________

VISIBLE FLUE LINER                    None                                                                                                                          NOT VISIBLE

Brick          Block          Clay          Concrete          Metal          Metal Insert          Rain Cap

Deterioration          Corrosion          Loose          Crack          Spalling          Improper          Advise Cleaning / Sweeping

May need a liner.

Contact a qualified technician to inspect the interior flue liner prior to purchasing the home. Clean and maintain chimney flues to promote safe and efficient operation of heating appliances and fireplaces.  This should be done annually or as prescribed by the chimney technician as obstructions can occur.

Chimney was to high and I could not see most of the flue pipe.

_______________________________________________________________________________________________________________

ATTIC

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

_______________________________________________________________________________________________________________

LIMITATION:          No Access          Sealed          Stored Items          Looked In          Entered          Hatch

Pull Down             Insulated

Adjust door to attic to operate better.

________________________________________________________________________________________________________________

STRUCTURE          Truss          Rafter                                                                                                                            DAMAGED—NO

Warped          Stain          Rot          Sag          Loose          Split

2X4 rafters. Looked ok.

________________________________________________________________________________________________________________

SHEATHING          Condensation                                                                                                                                    DAMAGED—NO

Composite          Thermal Board          Plywood          Board

Felt Paper          Mildew          Mold          Sag          Loose          Stain          Rot          Delaminated

No sheathing on roof. Cedar shake shingles are acting as the sheathing.

________________________________________________________________________________________________________________

INSULATION          Estimate Depth    6-8”                                                                                                                          FUNCTIONAL

Radiant Barrier          Concealed          Finished          None          Vapor Barrier          Fiberglass          Mineral          Cellulose          Foam

Wood Shavings          Rigid Plastic          Other          Batt          Blown          Loose          Sprayed          Required

Remove plastic of top of insulation so it can breathe.

Because of age of house there may not be any insulation in the walls.

________________________________________________________________________________________________________________

VENTILATION                    None                                                                                                                                        FUNCTIONAL

Soffit          Gable End          Turbine          Mechanical          Baffles          Roof          Ridge          Blocked          Required

________________________________________________________________________________________________________________

EXHAUST DUCT               Concealed                                                                                                                                                    N/A

Not Insulated         Into Attic          Plastic          Flex          Metal

________________________________________________________________________________________________________________

ELECTRICAL                 Concealed                                                                                                                           DAMAGED—NO

Abandoned          Knob & Tube          Open Slice          Frayed          Open Junction Boxes          Unsecured

ELECTRICAL SERVICE / PANEL

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

________________________________________________________________________________________________________________

For all electrical deficiencies noted below consult a qualified Licensed electrician to review and provide repairs, improvements and upgrades as needed.  These recommendations can impact safety in the home and should be addressed.

_______________________________________________________________________________________________________________

SERVICE ENTRANCE               Underground          Overhead          No Conduit          No Drip Loop

120-Volt          120/240 Volt          Unsecured          Frayed Service Line

ENTRANCE CABLE               Concealed          Aluminum          Copper

MAIN DISCONNECT               Switch / Cartridge Fuse          Breaker

DISCONNECT RATING                 100  Amps          Have Electrician Evaluate

Two old meters.

________________________________________________________________________________________________________________

DISTRIBUTION PANEL               Not Opened                                                                                                           DAMAGED—NO

LOCATION      BASEMENT                                                               Non Standard Installation

Obstructed          Unsecured          Corrosion          Obsolete

PANEL RATING                  100  Amps                     Room for Expansion

FUSE          Breaker         Glass / Screw         Cartridge         Time Delay         GFCI / AFCI Breaker         Blown / Broken          Over-Fused

2 panels. Water was seen in the basement panel. Some corrosion was seen on some breakers.  Have an electrician check further.

2nd panel on 1st floor looked ok.

Basement Panel.

Panel for 2nd floor.

________________________________________________________________________________________________________________

CIRCUIT WIRE          Improper                                                                                                                                      DAMAGED—NO

Aluminum          Copper          Copper Clad          Other

Non-Metallic Sheathed          Armoured Cable          Knob & Tube          Double Tapping          Spliced          Corrosion          Scorched

GROUNDING                     Concealed          Ground Rod          Water Main

Improper Connection / Installation          Corrosion          Unsecured

The integrity of the electrical system ground cannot be verified by a visual inspection.

BONDING                           Concealed          Water Pipe          Gas Pipe

Improper Connection / Installation          Corrosion          Unsercured

I recommend bonding the gas pipe to give it zero potential. Consult with an electrician.

_________________________________________________________________________________________________________________

BASEMENT / STRUCTURE

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

_______________________________________________________________________________________________________________

LIMITATION          Finished          Clutter          Obstruction          Dry Weather / Drought          Dry Ground

________________________________________________________________________________________________________________

FLOOR          Crack          Concrete          Dirt                                                                                                                  DAMAGED—YES

Carpet          Ceramic          Vinyl          Wood          Laminate          9” Tile

Floor was uneven and heaved in some areas. Repairs will be needed to level floor.

Basement structure conditions were concealed by stored items along walls and floor. 80 to 90 % concealed.

________________________________________________________________________________________________________________

WALL            Concrete          Block          Brick          Stone          PWF / Wood                                                             DAMAGED—NO

Mildew / Mold          Moisture Stains          Effervescence          Crack          Concealed

Water stains were seen on the walls. Water occasionally leaks in in heavy rain. I recommend having a waterproofing company check further. Some cracks in the block walls will need to be patched. Have a mason repair.

Repair crack.							Water stains.

__________________________________________________________________________________________

CEILING          Stain          Unfinished                                                                                                                             DAMAGED—NO

Drywall          Plaster          Wood          Fixed Tile          Drop Down Tile          Insulated          Uninsulated

________________________________________________________________________________________________________________

WINDOW          Binds          Not Tested                                                                                                                        DAMAGED—YES

Single / Double Hung          Casement          Awning          Sliding          Fixed          Bay / Bow

Thermal          Aluminum          Metal          Vinyl / Vinyl Clad          Wood

Damaged          Mildew / Mold          Stain / Rot          Repaint          Recaulk

Some rot was seen on basement windows. I recommend replacing.

_________________________________________________________________________________________

DOOR          Binds          Damaged                                                                                                                      OPERATIONAL—N/A

Pocket          Hinged          Wood          Composite

_______________________________________________________________________________________________________________

LIGHTING          Minimal          Unsecured                                                                                                         OPERATIONAL—YES

_______________________________________________________________________________________________________________

RECEPTACLE          Damaged          No Cover          Unsecured                                                                      OPERATIONAL—YES

Reverse Polarity          Install GFCI          No Ground          Open Ground

_______________________________________________________________________________________________________________

CIRCUIT WIRE          Concealed          Unsecured          Improper                                                                              DAMAGED—NO

_______________________________________________________________________________________________________________

HEAT SOURCE          None          Thermostat          Electric          Air Register          Convector          Radiator          Radiant

_______________________________________________________________________________________________________________

BASEMENT STAIRWAY          Unsecured                                                                                                               DAMAGED—NO

Carpet          Vinyl          Wood          Worn          Trip Hazard          Low Headroom

_______________________________________________________________________________________________________________

RAILING          Unsecured                                                                                                                                              DAMAGED—NO

Metal          Wood          Incomplete          None

Need a proper handrail for the stairs.

_______________________________________________________________________________________________________________

FLOOR JOIST          Concealed                                                                                                                                  DAMAGED—NO

Unsecured          Split          Notched          Stain          Rot          Other

Joists looked ok. Some sagging seen.

_______________________________________________________________________________________________________________

BRIDGING                  Concealed          None                                                                                                               DAMAGED—NO

Continuous          X-Metal          X-Wood          Solid Wood

_________________________________________________________________________________________

SILL PLATE                Concealed                                                                                                                                 DAMAGED—NO

Moisture Gasket          Termite Shield          Mildew / Mold          Stain / Rot          No Anchors

Sill plate looked ok in the main basement area.

Sill plate looked ok in the back crawlspace.

The sill plate was sagging and had some rot damage in the front crawlspace. It will need repair.

_______________________________________________________________________________________________________________

BEAM          Unsecured          Concealed                                                                                                                         DAMAGED—YES

Laminate          Metal          Wood          Sag          Notched / Bored          Insufficient End Bearing

Main beam was cracked and sagging. I recommend replacing.

_______________________________________________________________________________________________________________

POST          On Slab          Unsecured          Concealed                                                                                                    DAMAGED—NO

Adjustable          Brick          Block          Concrete          Metal          Wood

________________________________________________________________________________________________________________

BEARING WALL          Concealed                                                                                                                                DAMAGED—NO

________________________________________________________________________________________________________________

COLD ROOM            Improper Door                                                                                                                                                       N/A

Concrete          Brick          Block          Stone          Parged          Earth Floor          Mildew / Mold          Stain / Damp          Crack

________________________________________________________________________________________________________________

CRAWL SPACE          No Access          Limited Access                                                                                                 DAMAGED—NO

Vapor Barrier          Insulated          Insulation Upside Down          Entered          Looked In

Crack          Mildew / Mold          Stain / Rot          Damp          Water Stains          Sump Pump

Earth Floor          Concrete          Moisture Barrier Required

PIPES / DUCTS           Unsecured          Leak          Insulated          Uninsulated

VENTILATION             Mechanical          Blocked          Required

Back crawlspace looked ok. Add more insulation.

Front crawlspace needs to be cleaned out and a moisture barrier put down on the earth floor.

Also front wall will need to be supported better.

I recommend insulating between the floor joists.

Front crawlspace. Beam is where floor sticks up in front room. Hard to see but the sill plate has no support and is sagging on either side of the beam. Added support is needed.

________________________________________________________________________________________________________________

HEATING/COOLING/VENTILATION

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

_______________________________________________________________________________________________________________

DATA PLATE:        Not Legible        Incomplete        Model   IDEAL BOILER                   Btu Input  N/A                Est. Age  90 yrs

LIMITATIONS:          Clean-Out Does Not Open          Oil / Propane Tank Not Visible          System Operating in Heat / AC

System Shut-Off / Inoperative          Piping / Ducts Concealed          Weather / Temperature

________________________________________________________________________________________________________________

SMOKE DETECTORS          Basement          1st Floor          2nd Floor          3rd Floor          Other   None seen

CO DETECTORS                    Basement          1st Floor          2nd Floor          3rd Floor          Other   None seen

Provide NEW Carbon Monoxide sensors in bedroom areas and rooms with fuel burning appliances.

Recommend installation of NEW smoke detectors for safety of occupants.

________________________________________________________________________________________________________________

THERMOSTAT / HUMIDISTAT          Unsecured                                                                                        OPERATIONAL—YES

LOCATION     In living room.            Programmable          Standard

Replacing standard units with programmable thermostats will assist in reducing heating costs.

___________________________________________________________________________________________________

FUEL SOURCE          Gas / Propane          Oil          Wood / Pellet          Electric

HEAT TYPE                Convertor          Forced Air          Radiator

BURNER TYPE          Conventional          Mid-Efficiency          High-Efficiency

________________________________________________________________________________________________________________

HEATING SYSTEM          Advise Service / Repair Contract                                                                          OPERATIONAL—YES

AIR REQUIREMENTS          Internal          External          Inadequate

VENTING          Flue          Sidewall          Metal          Plastic

Improper Rises          Unsecured          Corrosion          Soot

LIFE EXPECTANCY          Typical          Middle          End          Exceeded

This is an oil fired, cast iron boiler, 2 zone hot water heating system.

The boiler is very old and replacement should be considered.

Advise Annual Preventive maintenance agreement from a local contactor.

Unit is showing signs relative to age and wear and is functioning as intended.

_________________________________________________________________________________________________________________

HOT WATER (w) / STEAM (s)                                                                                                DAMAGED—NO

BOILER CASE (w)                                 Deterioration               Corrosion              OK

CIRCULATING PUMP (w)                     Inoperative                  Corrosion              OK

AQUASTAT (w)                                     Missing                        Corrosion               Unsecured

BACK FLOW PREVENTOR                   Missing                       Corrosion               Leak

PRESSURE REDUCER (w)                  Missing                        Corrosion               Leak

EXPANSION TANK                               Location

Drain Tank               Open               Closed              Diaphragm

Waterlogged               Unsecured               Corrosion               Leak

PIPING (w/s)                                         Deterioration                 Corrosion               Leak

PRESSURE RELIEF VALVE                  Missing                       Corrosion               Leak

SITE GLASS (s)                                     Missing                        Corrosion               Low Water

LOW WATER CUTOFF (s)                    Missing                         Corrosion              Leak

PRESSURE CONTROLLER (s)            Missing                         Corrosion               Leak

Some signs of a material that looks like asbestos seen on pipes. Have tested to be sure.

Budget for removal

________________________________________________________________________________________________________________

OIL BURNER BLOWER          Not Checked                                                                                                    OPERATIONAL—YES

BURN CHAMBER          Deterioration          Advise Adjustment          Burn Through          Corrosion          Crack          Soot

INSPECTION DOOR          Missing          Soot          Sealed

BAROMETRIC DAMPER          Missing          Unbalanced          Sealed

OIL TANK / PIPING          Basement          Outside          Buried

No Filter          Unsecured          Corrosion          Kink          Leak

I recommend abandoning the buried oil tank.

________________________________________________________________________________________________________________

PLUMBING COMPONENTS

701 5th Ave., New Hyde Park, NY 11040                                     03/06/12    10:00 AM

_______________________________________________________________________________________________________________

LIMITATION          Finished Basement          Private / Water Shut Off          Private System         Cluttered Basement

________________________________________________________________________________________________________________

PUBLIC SUPPLY            Concealed                                                                                                                                    FUNCTIONAL

Lead          Galvanized          Plastic          Copper

SHUT-OFF VALVE          Location    BASEMENT – front wall

Not Tested          Corrosion          Leak

Water flow was acceptable at time of inspection.

________________________________________________________________________________________________________________

HOSE BIBB          Not Checked          Water Supply Off                                                                                                                      N/A

Frost Free          Anti-Siphon          Shut Off Valve          Recaulk          Unsecured          Corrosion          Leak

________________________________________________________________________________________________________________

DISTRIBUTION PIPING          Concealed                                                                                                                 DAMAGED—NO

Lead          Galvanized          Plastic         Copper         Dissimilar Metals         Unsecured         Corrosion         Leak

Corrosion was seen on several pipes at the connections. Some corrosion seen on the shutoff valves handles. Possible leak on elbow. Have a plumber repair.

________________________________________________________________________________________________________________

WASTE DRAINAGE              Concealed                                                                                                                       DAMAGED—NO

Galvanized / Steel          Cast Iron          Plastic          Copper

Odor          Unsecured          Corrosion          Leak                   Advise Cesspool Tank Checked / Pumped

Minor crack seen on the top of the cast iron waste pipe. This can be a future concern if the crack gets bigger. Have a plumber check further.

Galvanized steel water piping has not been used in homes since the late 1950’s early 1960's over time water passing through the pipes literally consumes the zinc once the zinc has worn away the exposed steel will rust from the inside out or close the diameter of the piping like a corroded artery reducing the water flow to the plumbing fixtures. There is no known health hazard associated with galvanized piping, certifying the condition of the piping is beyond the scope of this inspection.

FLOOR DRAIN          None          Backed-Up          No Water          No Trap

________________________________________________________________________________________________________________

VENT STACK / PIPING          Concealed                                                                                DAMAGED—NO

Galvanized / Steel           Cast Iron          Plastic          Copper

Undersized          Unsecured          Corrosion          Leak

MAIN CLEANOUT           Concealed                                                                                                                                                       N/A

Improper Plug                   LOCATION      NONE

May need to install a house trap. Consult with a plumber.

_______________________________________________________________________________________________________________

HOT WATER TANK          Hybrid Heating                                                                                                                 DAMAGED—NO

Power-Vented          Own          Rent          Est. Capacity    32                  Gas / Propane          Oil          Wood / Coal          Electric

Dirty           Rusty           Odor          Unsecured          Corrosion          Leak

LIFE EXPECTANCY           Typical          Middle          End          Exceeded

Unit is nearing end of normal life. Budget to replace in future.

FLUE SHUT-OFF          Concealed          Not Tested          Location

RELIEF VALVE          No Test Lever          Corrosion          Leak          Other

DISCHARGE TUBE          Undersized          Discharge          Missing

AGE:    9 Years

Unit appears in sound condition

________________________________________________________________________________________________________________

VENTING          Flue          Sidewall                                                                                                                                       FUNCTIONAL

Improper Rise          Unsecured          Corrosion          Soot

BURN CHAMBER           Not Checked            Needs Adjustment	Looked ok.

________________________________________________________________________________________________________________

WASHER          Make      Kenmore                                                                                                            OPERATIONAL—YES

DRYER             Make     Kenmore   Replace old recalled flex gas pipe with safer new one.

DRYER VENT          Unsecured                                                                                                                       OPERATIONAL—YES

With Other Exhaust          To Crawlspace          To Attic          To Exterior          Plastic Duct

Replace plastic duct with metal for fire safety.

Clean exhaust vent periodically, depending on use, but at least every 2 years, or when installing your dryer in a new location. Disconnect vent from the dryer and clean one section at a time until you reach the exhaust hood.

To clean out lint, use the hose attachment on your vacuum, or a pole or wire with a feather duster or rag

attached. Be sure the flapper on the outside end of vent moves freely. When cleaning is complete, be sure to follow the Installation Instructions supplied with your dryer for final product check."

________________________________________________________________________________________________________________

TWO  BATHROOMS

701 5th Ave., New Hyde Park, NY 11040                                     03/06/12    10:00 AM

_______________________________________________________________________________________________________________

LOCATIONS               Basement               1st Floor               2nd Floor               3rd Floor               Other

________________________________________________________________________________________________________________

WATER FLOW          Normal          Suspect          Low

________________________________________________________________________________________________________________

FLOOR          Worn          Crack          Stain                                                                                                                       DAMAGED—NO

Carpet          Vinyl          Wood          Laminate          Ceramic

________________________________________________________________________________________________________________

WALL          Patched          Unfinished                                                                                                                                DAMAGED—NO

Drywall / Plaster          Brick / Block / Stone          Wood          Wallpaper          Ceramic

________________________________________________________________________________________________________________

CEILING          Patched          Unfinished                                                                                                                          DAMAGED—NO

Drywall / Plaster          Drop In Tile          Fixed Tile          Wood

________________________________________________________________________________________________________________

WINDOW          Binds          Damaged                                                                                                                               DAMAGED—NO

Single / Double Hung          Casement          Awning          Sliding          Fixed          Bay / Bow

Thermo          Aluminum          Metal          Vinyl          Vinyl Clad          Wood

Damaged          Mildew / Mold          Stain          Rot          Repaint

________________________________________________________________________________________________________________

DOOR          Binds          Damaged                                                                                                                         OPERATIONAL—YES

Pocket          Bi-Fold           Hinged          Wood          Composite

________________________________________________________________________________________________________________

LIGHTING          None          Unsecured                                                                                                              OPERATIONAL—YES

________________________________________________________________________________________________________________

RECEPTACLE          Damaged          No Cover                                                                                                  OPERATIONAL—YES

Reverse Polarity          GFCI Receptacle          No Ground          Open Ground

________________________________________________________________________________________________________________

EXHAUST FAN          Advise Installation                                                                                                       OPERATIONAL—N/A

________________________________________________________________________________________________________________

SINK           Worn          Chipped          Scratch                                                                                                                  DAMAGED—NO

_______________________________________________________________________________________________________________

FAUCET          No Shut-Off                                                                                                        DAMAGED—NO

_______________________________________________________________________________________________________________

TRAP / DRAIN          Unsecured                                                                                                                                  DAMAGED—NO

Improper Trap          S-Trap          Slow Drain          Corrosion          Leak          Patched

_______________________________________________________________________________________________________________

VANITY          Worn          Unsecured                                                                                                                               DAMAGED—NO

Laminate          Plywood          Wood          Metal

Old vanity

_______________________________________________________________________________________________________________

COUNTER      Worn          Unsecured                                                                                                                                                 N/A

Solid Surface          Marble          Granite          Laminate          Ceramic

Regrout          Recaulk          Mildew          Stain          Scratch

_______________________________________________________________________________________________________________

TOILET               No Shut-Off                                                                                                                                             DAMAGED—NO

Tank Loose          Unsecured          Crack          Leak

First floor toilet does not flush well. Have a plumber check further.                                                                                                                                                                                                   _______________________________________________________________________________________________________________

TUB / ENCLOSURE              Unsecured                                                                                                                       DAMAGED—NO

Ceramic          Cult./Marble          Fiberglass          Plastic

Regrout          Recaulk          Mildew          Stain          Crack          Worn          Stopper Not Working

Clean and recaulk both tub areas.

_______________________________________________________________________________________________________________

JETTED TUB               Not Tested                                                                                                                                  DAMAGED—NO

GFCI Protected          Motor Access          No Motor Access          Corrosion          Leak

_______________________________________________________________________________________________________________

FAUCET / SHOWER HEAD          Not Tested                                                                                                        DAMAGED—NO

Sticks          Clogged          Unsecured          Corrosion          Leak

Old faucets. Budget to replace.

________________________________________________________________________________________________________________

SHOWER / ENCLOSURE              Unsecured                                                                                                                                  N/A

Ceramic          Cult./Marble          Fiberglass          Plastic

Regrout          Recaulk          Mildew          Stain          Crack          Worn

FAUCET / SHOWER HEAD              Not Tested                                                                                                                         N/A

Sticks          Clogged          Unsecured          Corrosion          Leak

________________________________________________________________________________________________________________

HEAT SOURCE          None          Thermostat          Electric          Air Register          Convector / Radiator          Radiant

TWO KITCHEN(S)

701 5th Ave., New Hyde Park, NY 11040                                03/06/12    10:00 AM

_________________________________________________________________________________________________________________

LOCATIONS               Basement               1st Floor               2nd Floor               3rd Floor               Other

_________________________________________________________________________________________________________________

FLOOR          Worn          Crack          Stain                                                                                                                        DAMAGED—NO

Carpet          Vinyl          Wood          Laminate          Ceramic

Floors are worn and uneven.

________________________________________________________________________________________________________________

WALL          Patched          Unfinished                                                                                                                                 DAMAGED—NO

Drywall / Plaster          Brick / Block / Stone          Wood          Wallpaper          Ceramic

________________________________________________________________________________________________________________

CEILING          Patched          Unfinished                                                                                                                          DAMAGED—NO

Drywall / Plaster          Drop In Tile          Fixed Tile          Wood

________________________________________________________________________________________________________________

WINDOW          Binds          Damaged                                                                                                                               DAMAGED—NO

Single / Double Hung          Casement          Awning          Sliding          Fixed          Bay / Bow

Thermo          Aluminum          Metal          Vinyl          Vinyl Clad          Wood

Damaged          Mildew / Mold          Stain          Rot          Repaint

________________________________________________________________________________________________________________

PATIO DOOR          Binds          Damaged                                                                                                                                            N/A

Metal / Vinyl          Wood          Hinged          Sliding ________________________________________________________________________________________________________________

LIGHTING          None          Unsecured                                                                                                           OPERATIONAL—YES

CEILING FAN          None          Unsecured                                                                                                                                         N/A

RECEPTACLE          Damaged          No Cover                                                                                                  OPERATIONAL—YES

Reverse Polarity          Install GFCI Receptacle          No Ground          Open Ground

Install GFCI receptacles for added safety.

SINK           Worn          Chipped          Scratch                                                                                                                  DAMAGED—NO

_______________________________________________________________________________________________________________

FAUCET          No Shut-Off                                                                                                       DAMAGED—NO

_______________________________________________________________________________________________________________

TRAP / DRAIN          Unsecured                                                                                                                                 DAMAGED—NO

Improper Trap          S-Trap          Slow Drain          Corrosion          Leak          Patched

________________________________________________________________________________________________________________

COUNTER                     Unsecured                                                                                                                                  DAMAGED—NO

Ceramic          Marble / Granite          Laminate          Solid Surface

Regrout / Recaulk          Mildew / Stain          Scratch          Worn

Second floor countertop looked ok. First floor was worn.

CABINETS                    Unsecured          Worn                                                                                                                DAMAGED—NO

Plywood          Wood          Laminate          Metal

Missing / Loose Hardware               Mildew / Stain          Scratch          Other

Cabinets were old and worn. New sink base on second floor looked ok.

_________________________________________________________________________________________________________________

RANGE HOOD             Cooktop Exhaust                                                                                                                        DAMAGED—NO

Corrosion          No Exhaust          No Light          Noisy

Add a range hood over second floor stove.

EXHAUST VENT                Unsecured          Ductless          Concealed                                                          OPERATIONAL—YES

With Other Exhaust            To Attic            Improper            To Exterior

FILTER           None          Unsecured          Damaged          Greasy

_________________________________________________________________________________________________________________

MAJOR APPLIANCES (Built In)

DISHWASHER                                                                                                               OPERATIONAL—N/A

STOVE / OVEN / RANGE                                                                                               OPERATIONAL—YES

REFRIGERATOR                                                                                                           OPERATIONAL—YES

Ice maker does not work and leaks water.

MICROWAVE                                                                                                                                            N/A

Tested On / Off                    Did Not Check All Functions / Full Cycle

_________________________________________________________________________________________________________________

HEAT SOURCE           None           Thermostat            Electric           Air Register           Convector / Radiator           Radiant

HOUSE ROOMS

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

________________________________________________________________________________________________________________

INTERIOR GENERAL COMMENTS

The interior consisting of the bedrooms, living and dining rooms, and other common spaces were similar in make up and materials and all comments will be listed in this section.   Specific conditions and remarks will identify the individual room in question (ie; Bedroom, Living Room ) and noted conditions should such exist at the time of the inspection.

The comments are intended to give an overview and highlight areas that may require maintenance, improvements, repairs or updating:

_________________________________________________________________________________________________________________

FLOOR          Worn          Crack          Stain                                                                                                                        DAMAGED—NO

Carpet          Vinyl          Wood          Laminate          Ceramic

Floors throughout the house were sagging and uneven. Added support in the basement will be needed.

________________________________________________________________________________________________________________

WALL          Patched          Unfinished                                                                                                                                 DAMAGED—NO

Drywall / Plaster          Brick / Block / Stone          Wood          Wallpaper          Ceramic

Some cracks were seen in the plaster walls.

________________________________________________________________________________________________________________

CEILING          Patched          Unfinished                                                                                                                           DAMAGED—NO

Drywall / Plaster          Drop In Tile          Fixed Tile          Wood

Some old water stains were seen.

________________________________________________________________________________________________________________

STAIRWAY           Unsecured                                                                                                                                           DAMAGED—NO

Carpet          Vinyl          Wood          Worn          Trip Hazard

RAILING          Unsecured                                                                                                         DAMAGED—NO

Metal          Wood          Incomplete          None

_______________________________________________________________________________________________________________

WINDOW          Binds          Damaged                                                                                                                            DAMAGED—YES

Single / Double Hung          Casement          Awning          Sliding          Fixed          Bay / Bow

Thermo          Aluminum          Metal          Vinyl          Vinyl Clad          Wood

Damaged          Mildew / Mold          Stain          Rot          Repaint

Old wood windows open ok where tested. Some panes were cracked. Some sashes were coming apart. They will need some maintenance or repair to get them into good condition. Add weep holes in the storm windows to remove excess water. Vinyl casement windows were ok.

________________________________________________________________________________________________________________

DOORS          Binds          Damaged                                                                                                                                DAMAGED—NO

Pocket          Bi-Fold           Hinged          Wood          Composite

Adjust some doors to stop binding.

________________________________________________________________________________________________________________

PATIO DOORS          Binds          Damaged                                                                                                                    DAMAGED—NO

Metal / Vinyl          Wood          Hinged          Sliding

Old wood doors. Adjustments are needed for doors to close better. Could use better weather stripping.

________________________________________________________________________________________________________________

CLOSET DOORS          Binds          Damaged                                                                                                               DAMAGED—NO

Metal / Vinyl          Wood          Hinged          Sliding

________________________________________________________________________________________________________________

LIGHTING          None          Unsecured                                                                                                                       DAMAGED—NO

CEILING FAN          None          Unsecured                                                                                                                DAMAGED—NO

RECEPTACLE          Damaged          No Cover                                                                                                               DAMAGED—NO

_________________________________________________________________________________________________________________

FRONT DOOR          Damaged                                                                                                                                    DAMAGED—NO

Binds          Metal Clad          Wood          Dead Bolt

Front door rubs on the floor because of settlement of the front exterior wall and the floor having a beam under it that did not settle.

_________________________________________________________________________________________________________________

HEAT SOURCE           None           Thermostat            Electric           Air Register           Convector / Radiator           Radiant

Additional Comments

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

____________________________________________________________________________________

General Comments

This inspection is performed to the Standards of Practice of New York State and is visual in nature, and does not address building code compliance issues which are the purview of municipal building inspectors.

Limitations

Chimney - The interior of chimneys and their flue liners are not visible on our visual inspection. You are advised to obtain the services of a qualified chimney sweeper or other qualified personnel to perform a complete inspection and tune up of your heating appliances prior to use.

Occupied Home – The home is occupied by the current owner with their personal belongings and furniture which limits some areas of inspection.

Circuit Sizing - The Inspector is required to address the compatibility of conductors and overcurrent devices. In some instances, general trade procedures include over-sizing overcurrent devices to guard against nuisance (e.g. air conditioning units, dryers). The Inspector is not required to evaluate such general trade procedures, but to inform you of incompatibility.

Cluttered Conditions - The home is in a cluttered condition with excess furniture, personal belongings, storage boxes, etc. throughout the home.  This limits the inspection and could conceal defects or damage that could otherwise be visible.

An opportunity to view the property prior to the closing with all excess items removed at the final walkthrough is customary and all areas should be checked at this time.

Vintage Homes - (homes built prior to 1950) - Character homes have inherent defects because they were built without the benefit of the standardized Building Codes. You must keep in mind that the defects noted throughout this report are typical of such dwellings because of normal wear and tear throughout the years. Often, maintenance repairs are ongoing and done based on your budget. If you tried to repair or improve all at once, costs could become high to the point where it is not practical or affordable to you at this time. Repairs or improvements which you would like to repair in the immediate future or have been advised to do, should be quoted prior to purchase by qualified contractors for costs and methods of repair.

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

____________________________________________________________________________________________________

Supplementary Comments

Not all receptacles/outlets tested due to limited accessibility (i.e. furniture, clutter and/or obstructions).

Not all windows or doors may have been checked due to obstructions (i.e. blinds, curtains and/or furniture).

Finished, insulated or cluttered basement prevents complete assessment of foundation, plumbing and electrical wiring.

It is very important that water & runoff drain away from foundations to minimize chance of water leakage into the basement, as cracks in foundation walls are common. Make sure the ground, patios and walkways slope away from the house for the first six feet.

Seal, caulk, repoint and/or repair cracks / gaps around the house (i.e. windows, sills, bricks & siding).

Wooden siding, windows, doors, decks, fences and/or sheds will require regular upkeep. Keep wood off the ground!

It is important that water from eve troughs drains well away from house - at least 6 feet away from wall. Also it is usually better to reposition downspouts to drain water above ground and away from house, then cap drains.

Make sure that eaves troughs remain fastened securely, gutters are kept clean seasonally from leaves & debris and leaks are sealed. Also, ensure that tree branches are not rubbing against eve troughs and roof.

Ensure roof flashing remains fastened, lays flat and gaps / cracks are caulked & sealed promptly to prevent leaks.

Monitor the roof and attic on a seasonal basis for leaks and (wind) damage. Repair as soon as possible.

Monitor siding and/or shingle exterior regularly for looseness, (wind) damage and repair and/or fasten as necessary.

Parging, stucco, brick and stone facing can crack, loosen and/or spall with age - monitor and repair as necessary.

Recommend installing & testing regularly Carbon Monoxide Detectors (locate in bedroom area + rooms with fireplace) and new Smoke sensors (mandatory every floor). Propane & Natural Gas sensors are also available in stores.

Older stone, brick & block foundations are porous. There could be dampness or leakage at certain times of the year.

Ensure that any cracks / gaps in bathroom or kitchen ceramic tiles are grouted, caulked and sealed to prevent leaks.

Some ceilings and walls may require nail holes, nail pops, dimples and/or cracks to be patched before painting.

Patched, stained, loose and/or uneven drywall / plaster noted - monitor and/or improve.

Some windows or doors require maintenance, upkeep (i.e. painting, weather stripping, caulking) and/or adjusting.

Some windows, doors screens and/or hardware are dated, older, missing or damaged - upgrade, repair or replace

Keep humidity low to prevent condensation, stains, mildew forming on windows & walls (<25% = -20 C; <40% = 0 C)

Consider refinishing wooden floors prior to moving in (Approx. $1.50 / sq. foor).

Some floors and/or carpets show significant wear and tear - improve as required.

Squeaking, uneven and/or sagging floors noted - monitor, improve as necessary.

Skylights frequently have condensation problems and/or can leak - monitor regularly

Cold room / cantinas frequently become damp and/or occasionally leak.

Ceramic floor tiles, especially larger ones, can crack unexpectantly or become loose - repair as required.

MAJOR COMMENTS - REPORT COMMENTARY

701 5th Ave., New Hyde Park, NY 11040                                       03/06/12    10:00 AM

Main areas of concern were:

Chimney will need repair.

Some regrading is needed.

Old wood windows will need repair or replacement.

Front exterior wall of the house will need better support.

Siding will need some repair.

Water problems in the basement will need correcting.

Old electric panels should be upgraded.

Need to keep water out of basement electric panel.

Budget to replace old boiler.

Budget to abandon buried oil tank.

Budget to remove asbestos on heating pipes if it is asbestos.

Old water heater. Budget to replace.

Have a plumber fix corrosion and leaks in water pipes.

Check crack in main waste pipe.

Replace cracked main beam.

Replace brass gas flex pipe for dryer.

Remove plastic dryer duct and install metal duct.

Sagging floors throughout the house will need better support.

Upgrade first floor Kitchen.

Upgrade bathrooms.